Supplementary Technical Note

Litvin Noncircular Conjugate Gear Pair – Requirements, Wear Distribution, and Piston-Motion Monitoring

Prepared for: Max Holden
Date: 2026-02-20

# 1. Scope and System Context

This note supplements the primary deep-research document by translating the project-specific discussion into engineering requirements, risk drivers, and measurable monitoring variables for a Litvin-derived noncircular, conjugate gear pair that drives an opposed-piston motion law. The focus is deliberately limited to the Litvin pair (planet-to-internal ring) and to the implications of: (i) forward-only torque, (ii) phase-locked tooth pairing, (iii) intentional distribution of contact across multiple ring teeth per planet tooth, and (iv) piston-motion tolerance and sensing as the primary “health metric.”

## 1.1 Operating envelope assumed in this supplement

Temperature: 100.000 °C to 300.000 °C (373.150 K to 573.150 K).

Engine speed: up to 7000.000 rpm (116.667 rev/s, 733.038 rad/s).

Average gear speed: approximately 14000.000 rpm (1466.077 rad/s) for two motion-law iterations per engine revolution.

Torque direction: single direction only (no reversal).

Kinematics: noncircular planets rolling/meshing against a noncircular internal ring surface; tooth pairing is synchronized (repeat contacts).

Life target: 10000.000 operating hours at/near the envelope above (≈ 4.200×10^9 engine revolutions; ≈ 8.400×10^9 average gear revolutions for 2× iteration).

Primary/secondary inertial forces: largely cancelled at the engine level by opposed-piston geometry; local mesh forces remain governed by gas load and mesh stiffness.

# 2. Litvin Pair: Functional Requirements and Design Constraints

A Litvin-style design/generation workflow for noncircular gears requires conjugate tooth flanks that satisfy enveloping-based kinematic compatibility across a phase-varying pitch radius. For this application, the gear pair is a motion-law generator. As a result, the design objective is not only strength but also long-term stability of the phase-resolved transmission error (TE) that maps directly to piston position.

## 2.1 Conjugacy and continuity requirements (motion-law integrity)

Conjugate flanks must preserve continuous rolling/meshing across the full operating phase range; avoid phase windows with abrupt contact condition changes (e.g., contact ratio or curvature discontinuities).

Mesh stiffness should vary smoothly with phase; discrete stiffness steps appear as piston-motion “kinks” even if average backlash remains small.

Microgeometry (lead crowning, profile modifications, relief) must be designed as a function of phase sensitivity, not as a single global involute-style prescription.

## 2.2 Manufacturing and metrology requirements (noncircular-specific)

Heat-treatment distortion control is critical for internal noncircular rings; process sequencing should explicitly include stress relief and post-HT finish operations.

Flank tolerances should be specified using a recognized classification system (e.g., ISO 1328-1) for baseline manufacturability, then tightened with a project-specific, phase-binned TE requirement. ISO 1328-1:2013 establishes a flank tolerance classification system for manufacturing and conformity assessment of individual cylindrical involute gear flanks (useful as a lingua franca even when the geometry is noncircular).

Inspection should include functional roll testing (TE measurement) under representative load and temperature, not only geometric inspection of individual flanks.

## 2.3 Materials/process notes tied to 100–300 °C

Because hardness, residual stress, and lubricant film thickness are all temperature-dependent, the material and surface engineering strategy must be compatible with sustained operation up to 300 °C. This supplement does not select a single alloy; instead, it defines what the material/process must achieve:

Stable surface hardness at operating temperature (resistance to softening).

High resistance to micropitting and scuffing in mixed lubrication (high PV segments).

Tough core for bending/root fatigue resistance under phase-localized peak loads.

Finish processes that enable low roughness and isotropic texture on loaded flanks (superfinish strongly favored).

# 3. Dominant Wear Patterns and Failure Modes in a Phase-Locked, Forward-Only Pair

Forward-only torque with synchronized tooth pairing produces deterministic, asymmetric wear: one flank of each tooth is consistently loaded and accumulates the majority of damage. Unlike a hunting-tooth involute pair, defects do not average out automatically; a single weak tooth-pair set can become the life limiter.

## 3.1 Key damage modes to manage

Micropitting: small surface fatigue pits that can reduce tooth accuracy and increase dynamic loads. ISO/TS 6336-22 provides a standardized procedure for micropitting load-capacity evaluation for cylindrical gears (useful as a reference framework for film-thickness-based risk control).

Scuffing (adhesive wear): promoted by high sliding speed, insufficient film thickness, and high flash temperature; often catastrophic compared to gradual micropitting.

Macropitting / pitting fatigue: progression from localized surface fatigue when stress/film conditions remain unfavorable.

Tooth-root bending fatigue: driven by peak loads (often near TDC-related phases).

Tooth flank fracture / case-cracking: especially relevant when high contact stress and brittle case conditions coincide; ISO/TS 6336-4 covers tooth flank fracture for case carburized gears (again as a reference framework).

Abrasive wear from contamination: tends to accelerate TE/backlash drift in deterministic pairs because the same tracks are repeatedly reworked.

## 3.2 “Kill-zone” concept (phase window that sets life)

For noncircular motion-law gearing, the life driver is typically a narrow phase window that combines: (i) high normal load, (ii) unfavorable sliding ratio, and (iii) reduced lubricant film thickness due to local speed/temperature. That window should be treated as a design feature: geometry, rim/web thickness, local microgeometry, and lubrication targeting should be optimized specifically for that phase range.

# 4. Distributing Contact: Multiple Ring Teeth per Planet-Tooth Set (Pseudo Hunting)

To mitigate deterministic wear localization without reversing torque, the system can be designed or controlled so that a given planet tooth engages a larger set of ring teeth over time. This is functionally analogous to a “hunting ratio,” where each pinion tooth contacts every gear tooth before repeating, but achieved here via motion-law iteration count and/or slow phase indexing.

## 4.1 Definitions

Define the following project variables:

k: number of motion-law iterations per engine revolution (k = 2.000 in the baseline two-stroke interpretation used earlier).

S_p: a specific planet tooth (or localized flank patch) being tracked.

T_r(S_p): the set of distinct ring teeth that contact S_p over a defined operational interval (e.g., one engine revolution, one hour, or a designed indexing cycle).

Cardinality |T_r(S_p)|: the number of distinct ring teeth in that set. Increasing |T_r(S_p)| increases wear averaging.

## 4.2 Levers to increase |T_r(S_p)| without torque reversal

Increase k (more motion-law repetitions per engine revolution): increases gear rotational speed and contact events, and can expand the ring-tooth population per planet tooth over a given engine-time interval.

Tooth-count strategy (hunting-like ratio): selecting effective tooth counts/phase mapping so that contacts do not repeat on short cycles; a hunting ratio concept ensures broad distribution before repetition.

Controlled slow phase drift (synthetic hunting): introduce a very slow, monotonic phase offset (e.g., via the sun gear as a tuning surface) so the contact path walks across different ring teeth over minutes/hours while remaining quasi-static over individual cycles.

## 4.3 Trade-offs and constraints (why this is not “free”)

Increasing k increases mean pitch-line speed and sliding velocity, raising scuffing risk unless lubrication film quality improves accordingly.

Higher rotational speed increases churning and windage losses, elevating bulk oil temperature and lowering viscosity (which reduces film thickness).

Synthetic hunting must be low-frequency relative to engine orders; otherwise it injects excitation into piston motion and can violate the motion-law tolerance.

The most practical implementation is typically a slow indexing schedule (hours-scale) rather than cycle-to-cycle modulation.

# 5. Piston Motion Tolerance and Its Relationship to Gear Wear

The system benefits from direct position sensing: gear-angle sensors (crank-like) and piston position sensing provide a closed-loop view of motion-law execution. Because the piston is kinematically linked to the planet center via a connecting rod, the measured piston position is the most direct measure of functional performance, even when tooth flanks wear.

## 5.1 Motion-law tolerance (functional)

Two equivalent tolerance statements were discussed. The more actionable one for diagnostics is a displacement-based limit:

| Tolerance statement | Example value | Diagnostic use |
| --- | --- | --- |
| Crank-angle deviation | ±5.000° crank rotation | Convenient for engine timing context; convert to x(θ) error for wear diagnosis. |
| Stroke fraction deviation | ≤0.500% stroke → 0.250 mm for 50.000 mm stroke | Directly maps to piston position sensor; easiest to trend vs time and phase. |

## 5.2 From piston error to equivalent transmission error (TE)

Let θ be crank angle, x_cmd(θ) the nominal piston motion law, and x_meas(θ) the measured piston position. Define the phase-resolved position error:

e_x(θ) = x_meas(θ) − x_cmd(θ)

To relate e_x to an equivalent mesh phase error (a TE-like variable), linearize the kinematic mapping around the nominal trajectory. If φ is the relevant gear-phase coordinate that drives the planet center/piston, then:

Δφ(θ) ≈ e_x(θ) / (dx/dφ)(θ)

Because (dx/dφ)(θ) is strongly phase-dependent for a noncircular motion-law, it should be precomputed from the design model and stored as a lookup table indexed by θ. The displacement tolerance e_x,max then implies a phase-dependent TE budget:

|Δφ(θ)| ≤ e_x,max / |(dx/dφ)(θ)|

## 5.3 Tooth-pair localization using phase-binned residuals

In a synchronized pair, a given crank-angle window corresponds to a consistent tooth-pair set. Therefore, persistent error patterns localized in θ can be mapped back to specific contact regions. A practical workflow is:

1. Compute e_x(θ) over many cycles and average/synchronously demodulate to obtain a stable phase signature.

2. Subtract predictable components due to elastic compliance and thermal drift (functions of load and temperature).

3. Use the residual signature e_res(θ) to identify which phase windows are degrading.

4. Map those windows to tooth-pair sets (planet tooth index, ring tooth index) using the known synchronization map.

# 6. Control Variables and Monitoring Strategy for a 10,000 h Life Target

With a 10,000 h target (≈ 10^9–10^10 repeat contacts, depending on phase and iteration count), design decisions should prioritize preventing fast, runaway damage modes (especially scuffing) and bounding gradual drift (micropitting/wear) within the piston-motion tolerance. Because the system is instrumented, the most valuable strategy is to treat wear distribution and calibration as controllable operating modes rather than as purely passive outcomes.

## 6.1 Control-variable set (what you can actually adjust)

| Control variable | Primary effect | Failure mode addressed | Notes/constraints |
| --- | --- | --- | --- |
| k (iterations per engine rev) | Distributes load and wear; changes gear speed and contact event rate | Wear localization, root stress | Raises sliding/churning; must be paired with upgraded lubrication. |
| Sun-gear phase indexing (slow drift) | Synthetic hunting: increases ¦T_r(S_p)¦ over hours | Deterministic wear tracks, debris rework | Keep low-frequency to avoid injecting piston excitation. |
| Oil temperature setpoint | Controls viscosity → film thickness (lambda) and scuffing margin | Scuffing, micropitting | Requires stable cooling and adequate flow/jetting. |
| Filtration & cleanliness level | Reduces abrasive wear and micropitting initiation sites | Abrasive wear, micropitting | Set maintenance intervals via debris monitoring if possible. |
| Microgeometry bias schedule | Keeps load distribution favorable in high-stress phase windows | Micropitting/macropitting, TE growth | Implemented via manufacturing (static) or via a controlled phasing schedule (dynamic). |

## 6.2 Monitoring outputs (what to log and threshold)

Phase-binned piston position residual e_res(θ): primary health signal for motion-law fidelity.

Derived equivalent phase error Δφ(θ): TE-like metric that normalizes for phase sensitivity via (dx/dφ)(θ).

Trend of peak-to-peak e_res in the high-load phase window (near TDC-related segments): should remain within the 0.250 mm budget with margin.

Temperature and inferred load proxies (e.g., indicated mean effective pressure or torque estimate): required to separate wear from elastic/thermal effects.

Optional: vibration/structure-borne signatures correlated to TE steps (useful for early scuffing detection).

## 6.3 Recommended control logic (high-level)

1. Run in a baseline mode with fixed phasing to establish a healthy reference signature e_ref(θ).

2. Apply slow phase indexing only when e_res(θ) indicates localized growth (aim: spread wear rather than fight it).

3. Maintain oil temperature/flow such that scuffing margin is protected in the kill-zone phase window.

4. Trigger maintenance/inspection actions based on trend thresholds in e_res(θ) and/or debris metrics, rather than on hours alone.

# 7. Implementation Checklist (Design → Manufacture → Validation)

## 7.1 Design and analysis

Generate conjugate noncircular flanks using the Litvin methodology and validate conjugacy numerically across full phase range.

Compute phase-resolved: normal load, sliding ratio, mesh stiffness, predicted film-thickness proxy; identify kill-zone window(s).

Design local microgeometry and structural reinforcement (ring rim/web, face width distribution, lead correction) to protect the kill zone.

Specify a phase-resolved TE budget derived from piston displacement tolerance and (dx/dφ)(θ).

## 7.2 Manufacturing and inspection

Define process chain with explicit distortion control and post-HT finishing for the internal ring.

Specify flank tolerances using ISO 1328-1 classes as baseline language, plus project-specific TE acceptance criteria.

Perform functional TE roll testing under representative load and temperature whenever feasible.

## 7.3 Instrumentation and validation

Install gear-angle sensors and piston position sensing with synchronized timestamping.

Implement the observer: x_cmd(θ), x_meas(θ), e_x(θ), Δφ(θ), and phase-binned trending.

Validate separation of wear vs temperature/load effects via controlled sweeps (temperature and torque).

Run an accelerated test campaign to provoke micropitting/scuffing boundaries and confirm the control strategy protects against runaway modes.

# References (selected)

ISO 1328-1:2013, Cylindrical gears — ISO system of flank tolerance classification — Part 1: Definitions and allowable values of deviations relevant to flanks of gear teeth. ISO. (Published 2013).

ISO 6336-1:2019, Calculation of load capacity of spur and helical gears — Part 1: Basic principles, introduction and general influence factors. ISO. (Published 2019).

ISO/TS 6336-22:2018, Calculation of load capacity of spur and helical gears — Part 22: Calculation of micropitting load capacity. ISO. (Published 2018).

ISO/TS 6336-4:2019, Calculation of load capacity of spur and helical gears — Part 4: Calculation of tooth flank fracture load capacity. ISO. (Published 2019).

Litvin, F. L., Fuentes-Aznar, A., Gonzalez-Perez, I., & Hayasaka, K., Noncircular Gears: Design and Generation. Cambridge University Press. (2009/2014 editions).

Gear Technology (article): “Noncircular Gears” (overview of Litvin noncircular gearing literature and applications).

Machine Design (article): “Gearing basics” (includes definition of hunting ratio and related terminology).